# RF Frequency Analysis Hardware Kit

Medium Grade - Off-the-Shelf Components

Date: March 23, 2026

# Table of Contents

1. Executive Summary								3

2. Hardware Tier Comparison								4

3. Recommended Kit Configuration								5

4. Phased Implementation Plan								6

5. Software Stack								7

6. Frequency Analysis Workflow								8

7. Budget Summary								9

8. Learning Resources								10

9. Safety and Legal Considerations								11

10. Conclusion								12

# 1. Executive Summary

This document outlines a comprehensive roadmap for building a medium-grade radio frequency (RF) analysis hardware kit using off-the-shelf components. The kit is designed for users transitioning from beginner to intermediate level, providing professional-grade capabilities at reasonable cost.

The recommended configuration centers on the HackRF One, providing full duplex (TX+RX) capability across 1 MHz to 6 GHz with 20 MHz instantaneous bandwidth. Total investment of $1,000 provides professional-grade analysis capability with a phased implementation approach.

## Key Specifications

| Specification | Value |
| --- | --- |
| Frequency Range | 1 MHz - 6 GHz |
| Bandwidth | Up to 20 MHz |
| TX Power | 0 to +15 dBm |
| Sample Rate | 20 MSPS (8-bit I/Q) |
| Total Budget | $1,000 (phased) |
| Implementation | 16 weeks |

# 2. Hardware Tier Comparison

## Tier 1: Entry Level (Beginner)

| Device | Frequency | Bandwidth | Price | Best For |
| --- | --- | --- | --- | --- |
| RTL-SDR V3 | 24-1766 MHz | 2.4 MHz | $35 | Learning |
| RTL-SDR V4 | 500k-1.7 GHz | 2.4 MHz | $45 | HF + VHF/UHF |
| Nooelec SMArTee | 24-1766 MHz | 2.4 MHz | $40 | Low noise |

## Tier 2: Medium Grade (Intermediate) - RECOMMENDED

| Device | Frequency | Bandwidth | TX Power | Price |
| --- | --- | --- | --- | --- |
| HackRF One | 1-6000 MHz | 20 MHz | 0-15 dBm | $300 |
| LimeSDR | 100k-3.8 GHz | 61.4 MHz | Up to 10 dBm | $300 |
| LimeSDR Mini | 10M-3.5 GHz | 30.7 MHz | Up to 10 dBm | $150 |
| BladeRF x40 | 300M-3.8 GHz | 28 MHz | Up to 6 dBm | $400 |

## Tier 3: Professional Grade (Advanced)

| Device | Frequency | Bandwidth | Price |
| --- | --- | --- | --- |
| Ettus USRP B210 | 70M-6 GHz | 56 MHz | $1,100 |
| BladeRF 2.0 micro | 47M-6 GHz | 56 MHz | $540 |
| PlutoSDR | 70M-6 GHz | 56 MHz | $230 |

## Frequency Coverage Comparison

Figure 1: Frequency range comparison across SDR devices.

## Price vs Bandwidth Comparison

Figure 2: Price versus bandwidth for different SDR platforms.

# 3. Recommended Kit Configuration

## Primary Device: HackRF One ($300)

| Specification | Value |
| --- | --- |
| Frequency Range | 1 MHz - 6 GHz |
| Operating Modes | Half-duplex (TX or RX) |
| RF Bandwidth | Up to 20 MHz |
| Sample Rate | 20 MSPS (8-bit I/Q) |
| TX Power | 0 to +15 dBm typical |
| RX Sensitivity | -80 dBm typical |
| Interface | USB 2.0 |
| Antenna Connector | SMA female |

Why HackRF One: Widest frequency range (1 MHz - 6 GHz), transmit AND receive capability, massive community support, open source hardware, excellent documentation and tutorials.

## Supporting Equipment

| Category | Items | Budget |
| --- | --- | --- |
| Antennas | Dipole kit, Discone, Log Periodic | $100-150 |
| Cables/Adapters | SMA cables, BNC/N adapters, Attenuators | $50 |
| Accessories | USB hub, Case, LNA, Bias-tee, TCXO | $80-150 |
| Computing | Laptop (i5+, 8GB RAM, USB 3.0) | $200-500 |

## Capability Comparison

Figure 3: Radar chart comparing device capabilities across key metrics.

# 4. Phased Implementation Plan

Figure 4: 16-week implementation timeline with milestones.

## Phase 1: Foundation (Weeks 1-4) - $100

| Equipment | Cost |
| --- | --- |
| RTL-SDR Blog V4 dongle | $45 |
| Basic dipole antenna kit | $25 |
| USB cable, SMA adapters | $30 |

Skills: Software installation, frequency scanning, signal identification, recording

## Phase 2: Enhancement (Weeks 5-8) - $400

| Equipment | Cost |
| --- | --- |
| HackRF One | $300 |
| Discone antenna | $80 |
| USB 3.0 hub | $25 |

Skills: Transmitting signals, GNU Radio basics, modulation analysis

## Phase 3: Analysis (Weeks 9-12) - $200

| Equipment | Cost |
| --- | --- |
| LNA with bias-tee | $60 |
| Better cables and adapters | $50 |
| Directional antenna | $60 |
| TCXO upgrade | $30 |

Skills: Weak signal reception, direction finding, regulatory compliance

## Phase 4: Advanced (Weeks 13-16) - $300

| Equipment | Cost |
| --- | --- |
| Second SDR (diversity) | $45 |
| Shielded enclosure | $50 |
| GPSDO for timing (optional) | $200 |

Skills: Coherent reception, advanced GNU Radio, security testing

# 5. Software Stack

## Core Analysis Software

| Software | Platform | Purpose | License |
| --- | --- | --- | --- |
| GQRX | Linux/Mac | General receiver | GPL |
| SDR++ | Cross-platform | Wideband receiver | GPL |
| SDR-Angel | Windows/Linux | Multi-mode receiver | GPL |
| CubicSDR | Cross-platform | Visual spectrum | BSD |
| GNU Radio | Cross-platform | DSP framework | GPL |

## Specialized Tools

| Software | Purpose |
| --- | --- |
| URH (Universal Radio Hacker) | Protocol analysis |
| SigDigger | Signal analysis |
| Baudline | Spectral analysis |
| Fosphor | GPU-accelerated waterfall |
| QSpectrumAnalyzer | Python spectrum analyzer |

# 6. Budget Summary

Figure 5: Phased budget breakdown with cumulative costs.

| Phase | Equipment | Cost |
| --- | --- | --- |
| Phase 1 | RTL-SDR kit | $100 |
| Phase 2 | HackRF One + antenna | $400 |
| Phase 3 | LNA, adapters, directional | $200 |
| Phase 4 | Second SDR, shielding | $300 |
| Total |  | $1,000 |

## Budget Options

| Configuration | Total Cost | Capability |
| --- | --- | --- |
| Minimum Viable | $450 | RTL-SDR + HackRF |
| Medium-Grade | $1,000 | All 4 phases |
| Optimal | $1,500 | With upgrades |

# 7. Learning Resources

## Free Resources

| Resource | URL | Focus |
| --- | --- | --- |
| RTL-SDR.com | rtl-sdr.com | Beginner tutorials |
| Great Scott Gadgets | greatscottgadgets.com/sdr | Video series |
| GNU Radio Tutorials | tutorials.gnuradio.org | DSP fundamentals |
| Signal Identification Wiki | sigidwiki.com | Signal database |
| Radio Reference | radioreference.com | Frequency database |

## Recommended Books

| Book | Author | Price |
| --- | --- | --- |
| SDR for Engineers | Analog Devices | Free PDF |
| GNU Radio Cookbook | Various | Free online |
| Signals and Systems | Oppenheim | $120 |

# 8. Safety and Legal Considerations

## Transmit Restrictions

| Band | TX Allowed? | License Required |
| --- | --- | --- |
| AM Broadcast | NO | N/A |
| FM Broadcast | NO | N/A |
| Amateur Radio | YES | Amateur license |
| ISM Bands (315/433/915 MHz) | YES | Part 15 compliant |
| Citizen's Band | YES | None (Part 95) |
| FRS/GMRS | Partial | GMRS requires license |

## Best Practices

1. Never transmit on restricted frequencies

2. Use attenuators when testing near transmitters

3. Obtain proper licenses before TX

4. Keep records of all transmissions

5. Respect privacy - do not decode private communications

# 9. Conclusion

A medium-grade RF analysis kit centered on the HackRF One provides excellent value for intermediate users. The phased approach allows gradual skill development while building capability. Total investment of $1,000 provides professional-grade analysis capability across 1 MHz to 6 GHz with both transmit and receive functionality.

Key recommendations:

Start with RTL-SDR for learning ($45-100)

Upgrade to HackRF One for TX capability ($300)

Add LNA and better antennas for weak signals ($100)

Expand to multi-SDR setups for advanced analysis ($300+)

This kit provides the foundation for RF spectrum analysis, signal identification, protocol reverse engineering, security testing (authorized), amateur radio operation, and regulatory compliance verification.

# Appendix: Vendor List

| Vendor | Products | Website |
| --- | --- | --- |
| RTL-SDR Blog | RTL-SDR V3/V4 | rtl-sdr.com |
| Nooelec | NESDR series | nooelec.com |
| Great Scott Gadgets | HackRF | greatscottgadgets.com |
| Lime Microsystems | LimeSDR | limemicro.com |
| Nuand | BladeRF | nuand.com |
| Ettus Research | USRP | ettus.com |